# Groundwater pumping limits in the Central Valley — COEQWAL Findings — Copy Deck for Language Review

Source: strategy_brief_groundwater-pumping-limits-in-the-central-valley_v0_1.html

This is a background brief — it reports no modeled results. The values in the text (scenario identifiers, configuration settings, and percentages) are fixed facts about the scenario, not data that will be regenerated. Edit the wording, not those values.

## How to use this document

This holds only the words a reader sees in the brief, block by block, each with a label that matches a spot in the page. To suggest wording changes, turn on Track Changes (Review tab → Track Changes) and edit the text directly. Accepted edits are then applied back to the brief's HTML.

Edit wording only. Numbers, tier values, and counts are inserted from data — leave them as-is; they will be regenerated. The block labels and section headings are signposts, not part of the brief; don't edit those.

Every brief keeps all nine sections in fixed order, including any that don't apply (kept with a short note). Please keep that structure.

Front matter

Masthead (top strip)

COEQWAL Strategy Brief · Background

Eyebrow (sector)

Operational Strategy · Farms and groundwater

Headline (H1)

Groundwater pumping limits in the Central Valley A management strategy in the COEQWAL scenario library

Headline / pull-quote (TL;DR)

This strategy adds monthly limits on groundwater pumping for demand units across the Central Valley, set at levels estimated to minimize long-term groundwater storage decline. Everything else matches current operations.

Identity sentence

This brief describes groundwater pumping limits in the Central Valley, one of 24 management strategies in the COEQWAL scenario library. Each strategy is simulated once under each of five hydroclimates; the references note at the end of this brief lists the model runs.

Search / meta description

A COEQWAL strategy background brief describing groundwater pumping limits in the Central Valley, one of 24 management strategies in the COEQWAL scenario library: monthly groundwater pumping limits for demand units across the Central Valley. Background only — no modeled results.

## §2 What this strategy changes

¶1

The strategy assigns limits on groundwater pumping in demand units across the Central Valley portion of the CalSim3 domain. The limits are set according to estimates of the pumping reduction needed to minimize groundwater storage decline over the full 100-year simulation period, and are implemented in the model code on a monthly basis for each demand unit as a fraction of its applied water demand.

¶2

When surface deliveries and the pumping limit together fall short of a demand unit's applied water demand, the remainder appears as a shortage for that unit in that timestep. For example, if a unit's pumping limit is 60% of an applied water demand of 10 TAF, and surface-water deliveries total 3 TAF, groundwater pumping can supply at most 6 TAF and the remaining 1 TAF is recorded as a shortage.

¶3

CalSim3 assumes all irrigation demands are met in every timestep, an assumption that carries into the soil-water budget of the CalSimHydro pre-processing model. Because limiting pumping outside that demand calculation violates the assumption, deep percolation from applied irrigation water is adjusted to reflect the shortages the pumping limits create, using empirical relationships between deep percolation and applied water in summer months derived from the current-operations configuration.

¶4

The configuration is meant to mimic, in part, groundwater management actions that may occur under the Sustainable Groundwater Management Act (SGMA). It should not be read as a direct or detailed implementation of that process as it exists now or as it may evolve.

## §3 The configuration in detail

¶1

The table compares the eight operational fields that define this strategy against current operations — the strategy that represents existing operational rules, recent (2020) land use, and TUCPs. Rows that match current operations are shown in light gray; rows that differ are highlighted.

## §4 Lineage and source

¶1

This strategy was developed by the COEQWAL team from current operations. The change is the addition of groundwater pumping limits for Central Valley demand units, with associated adjustments to weighting, shortage terms, and irrigation water-budget corrections. The CalSim model files for this strategy are referenced by the run IDs in the references note at the end of this brief.

## §5 Related strategies

Subheading

Related strategies — Farms and groundwater

List item

Groundwater pumping limits in the San Joaquin Valley. Monthly groundwater pumping limits for San Joaquin Valley demand units, set at levels estimated to minimize long-term groundwater storage decline.

List item

Groundwater pumping limits and reduced crop acreage in the San Joaquin Valley. Reduced irrigated crop acreage in the San Joaquin Valley, lowering irrigation demands and indirectly limiting groundwater pumping.

List item

Groundwater pumping limits and reduced crop acreage in the Central Valley. Reduced irrigated crop acreage across the Central Valley, lowering irrigation demands and indirectly limiting groundwater pumping.

List item

Functional environmental flows with groundwater regulations. Functional flow requirements combined with reduced irrigated acreage across the Central Valley, reflecting groundwater management under SGMA.

List item

Salmon-friendly flows with groundwater regulations. The salmon-friendly flows combined with reduced irrigated acreage across the Central Valley, reflecting groundwater management under SGMA.

## §6 What this doesn't say

¶1

This brief describes an idealized representation in CalSim, not a real-world policy proposal or implementation plan. The strategy as modeled may not capture all details necessary to support direct translation to actual operations. The brief does not predict, evaluate, or recommend any outcome — it describes what the scenario changes about CalSim's representation of the system. For what the model produces under this strategy, see the result briefs referenced in §7.

## §7 See results for this strategy

¶1

To see modeled outcomes under this strategy across all nine sectors at the reference hydroclimate, see the Farms and groundwater theme brief.

¶2

To see outcomes at a specific place, see the relevant LOI briefs — particularly those for groundwater and agricultural locations across the Central Valley.

Explorer link

Compare this strategy in the data explorer →

## References — model runs

¶1

Each strategy is simulated once under each of five hydroclimates. The table lists the model run behind this strategy and the current-operations run it is compared against, for each hydroclimate, so any reader can trace this page to specific CalSim runs.